MRV266 Firmware Program Release Notes

V4.6.2.0

New Features:

- Added the calibration acceleration function to increase the speed of stably uploading calibration coefficients.

Improvements:

- Improved the loading capacity up to 512×384 pixels.

Changes:

- Removed the function of setting of pre-stored image.

- Removed the chroma calibration function.

- Removed the function of quickly uploading calibration coefficients.

Notice:

- To realize the loading capacity, use the receiving card with NovaLCT V5.3.0 or later.

- To use the calibration acceleration function, use the receiving card with the controller that supports this function.

V4.6.1.0

Bug Fixes:

- Fixed the problem of color temperature adjustment function.

V4.6.0.0

Functions:

- Loading capacity: 512×256 pixels

- Support for up to 24 groups of parallel data or 64 groups of serial data (expandable to 128 groups of serial data)

- Support for at most 64 scans

- Voltage and temperature monitoring

- Mapping function

- Pixel level brightness and chroma calibration

- Quick adjustment of dark or bright lines

- Individual Gamma adjustment for RGB

- Setting of pre-stored image in receiving card

- Bit error rate monitoring

- Support for a 5-pin LCD module

- 3D function

- Configuration parameter backup and readback

- Firmware program readback

- Loop backup

- Dual backup of program

Driver Chips:

- MBI series: MBI502x, MBI503x, MBI5041(B), MBI5042(B), MBI5043, MBI505x, MBI5124 (not including MBI5124DPWM), MBI5125 (not including MBI5125DPWM), MBI515x, MBI5252, MBI5353, MBI5353B

- SUM series: SUM20167, SUM2017(T), SUM2028, SUM203x, SUM213x

- MY series: MY9266, MY9269, MY9366, MY9862, MY9868

- ICN series: ICN2027, ICN2028, ICN2038, ICN2038S, ICN2053, ICN2046, ICN2055, ICN2065

- SM series: SM16158, SM16159, SM16207, SM16227, SM16237, SM16259

- Others: TLC5958, TLC59581, SC8060, common chips

Decoding Chips:

- 74HC138 chip

- 74HC595 chip

- RT5953 and RT5958 chips

- SM5266 and SM5366 chips

- ICN2012, ICN2013, ICN2018, and ICN2019 chips